PROGRAMMING

1) You have a message "Hi".  It is a correct and proper null terminated character array.  What's the smallest array of chars that can hold this word?
    0     1      2       3       4       5      6       7       8      more

2) What will this program print?

- main() {

- int a = 1;

- int pid = fork();

- pid = fork();

- pid = fork();

- if (pid > 0)

- cout << a << endl;

- }

3) Suppose that the server sends a four byte message consisting of "1" "2" "3" and "4".  What will the receiving code below print?  Be careful!

char buf[100];

strcpy(buf, "abcdefg");

read(sock, buf, 4);

cout << buf << endl;

4) In ten words or less, what does poll() do?

5) Which of these looks like the best server code?  You must pick ONE.

| pid = fork() fd = accept ( ...) if (pid == 0) handle connect exit | fd = accept( ..) pid = fork() if (pid == 0) handle connection exit | while(1) pid = fork() fd = accept( ... ) if (fd == 0) handle connection exit | while(1) fd = accept( .. ) pid = fork() if (pid == 0) handle connection exit |
| --- | --- | --- | --- |

ETHERNET

1) Suppose that an Ethernet packet is sent by the sender without error or collision, but the receiver never receives it.  Will the ethernet card retransmit the packet?

  - Yes

  - No

  - Depends on the type of airplane

2) (True/False) Ethernet uses IP addresses to route packets.

3) (True/False) An Ethernet card will either deliver correct data, or no data.  An Ethernet card will almost never offer corrupted data to it's host computer.

4) (True/False) A normally, correctly operating Ethernet will have collisions.  That's OK.

OTHER

1) In our lab, which computer is the ARP server? __________________________________

2) I sent an email.  Ten hours later the reader reads it.  Where was it during these ten hours?

In my computer
     In the receiver's email server
     In the cloud

3) BGP does not produce optimal routes.  Why?

TCP/UPD/IP

1) Put a "T" next to those programs that probably use TCP, and a "U" next to those programs that probably use UDP

  - ___ HTTP (web pages)

  - ___ Windows Media Player (streaming audio)

  - ___ Kazaa (transfers MP3s)

  - ___ Call of Duty (the part that sends the map from server to client)

1) Which of the following system calls place bytes upon the wire?  (circle any that apply)

- socket  connect   write   read    gethostbyname    close   bind   accept

2) How many packets does it take to start a TCP connection?

0      1      2       3       4      5       6      7      8     more

3) I send some bytes using TCP.  Which is true

The data might fail to be received, and yet I get no error.

If the data is not received, I WILL get an error.

4) Using IP, who routes the packets?

The sender chooses the route

Each router along the way chooses the next hop.

HTTP

1) I want to download a small file one time. I have no more requests for this server.  Which protocol will send the data in the least number of bytes?

- HTTP/0.8     HTTP/0.9     HTTP/1.0     HTTP/1.1    HTTP/1.2

2) I'm using HTTP/1.0.  Everything works great.  The web server gets a request.  What is the first byte that the server sends when it sends a normal file in the normal way from the server to the browser?

WRITE ME CODE ON THE COMPUTER.  I WANT TO SEE IT RUNNING ON THE COMPUTER.

(5 points) Make me a port scanner.  It tries to connect to every port from 1 … 1024 and tells me which ones work.  Most ports have no one listening, but some (like port 80) do.  Make me a list of those that do.

--OR--

(3 points)  Make me a program that goes into a loop reading.  It replies with the number of bytes read, and prints those bytes to standard output.

--OR--

(1 point) Make me a program that tells me if there is a server running on turing.nmu.edu port 111.  I just want to know if that port has a server on it.