HTTP

1) Firefox has a setting browser. cache. check_doc_frequency entitled "Document is cache is compared to document on network".  The choices are "Every time", "Once per session", and "Never". For instance, if "every time" is set then the cached document is compared to the servers version every time the document is displayed to the user. If the documents differ, the new document is loaded. What would be the most efficient way to implement this feature?

Use a 'Head' command where possible 
	Use a "Head" command with a 'Send-if-modified' if possible 
	Use a "Get" command with a "If-Modified-Since" if possible 
	Use a "Get" command with a "Send-date" or "Send-Etag" modifier if possible 
	None of these. It's actually ________________________________

2) You want everything to go right.  What return code do you hope for from the server?

- Something in the 100's

- Something in the 200's

- Something in the 300's

- Something in the 400's

- Something in the 500's

3) Using HTTP/1.1, how can you tell what type the incoming file is?
______________________________________________________________________________

4) Suppose you make an HTTP/1.0 request.  You receive data in reply.  How can you tell where the metadata stops and the file contents begin?

- Programming

1) As a receiver I execute the code ret = read(sock, buf, 1000);  However, the sender only sends 28 bytes and then closes the socket and then exist.  What value will I get for 'ret'?

2) Spock has 3 piles of Mars sand.  Kirk has 4 piles of Mars sand.  If they put them together, how many piles do they have?  ______________

3)  Consider the two programs below. What happens if I launch ONE server and the try and start TWO clients at nearly the same time?

| main() { struct sockaddr_in sa; int sa_len; char hostname[1024]; sa.sin_port = htons(7777); gethostname(hostname, 1024); struct hostent *hp = gethostbyname(localhostname); bcopy((char *)hp->haddr, (char *)&sa.sin_addr, hp->h_length); int sock = socket(hp->h_addrtype, SOCK_STREAM, 0); bind(s, (struct sockaddr *)&sa, sizeof(sa)); listen(s, 99); int fd = accept(s, (struct sockaddr *)&sa, (unsigned int *)sa_len); sleep(3600); // wait for one hour write(fd, "Hi\0", 3); } | main() { struct sockaddr_in sa; int sa_len; sa.sin_port = htons(7777); struct hostent *hp = gethostbyname("euclid.nmu.edu"); bcopy((char *)hp->haddr, (char *)&sa.sin_addr, hp->h_length); int sock = socket(hp->h_addrtype, SOCK_STREAM, 0); int fd = connect(sock, (struct sockaddr *)sa, sizeof(sa)); read(fd, buf, 1024); cout << buf << endl; } |
| --- | --- |

4)What type of protocol does this program use?  (Pick ALL that apply) 
   TCP    UDP     IP     Aloha

5) Assume I run one client and one server from above, and no errors occur.  Which of the following is true?  (Pick exactly ONE) 
	a) When the client finishes the connect call, the server has finished accept. 
	b) When the server finishes the socket() call, the client must have finished it's socket() call. 
	c) When the client is at connect(), the server is at bind(). 
	d) Both the server and client will run gethostbyname at the same time.

6) With what type of problems does  one use fork() to handle multiple clients?  What is a good indication that the fork() technique will work well?

TCP/UDP/IP

1) Suppose I start a TCP connection.  I receive bytes 0 .... 1000, then 1001 ... 2000, then 5000 .. 6000.  What byte number should my last ack contain?

2) Suppose I am a TCP sender and a timeout occurs without receiving an ack.  What should do?  List two things?

______________________________________________________________________

_______________________________________________________________________

3) Suppose I am a TCP receiver and I receive a packet containing repeat data.  What should I do?

4) How many packets does it take to send a small HTTP request and get a small reply.  Count all packets.

5) (Source/Not Source) General Hospital is going to be attached to the Internet.  Do computers attached to the internet generally choose the whole route when the packet is sent (source routing) or just choose the link to send the packet out on, trusting that the next computer in line can send it closer still to the destination

6) Lost TCP ack's do not necessarily force a retransmission. Explain how this can happen?   __________________________________________________________________

__________________________________________________________________

__________________________________________________________________

7) Which is true about these protocols.  Put TCP or UDP next to each attribute.
	Lower latency. 
	Larger header. 
	Retry after failure. 
	Used by HTTP

8) Suppose I want to send 50 bytes of data. The transmission is successful. Ignoring any ack's or ARPs, how many bytes does each layer send?
	TCP _________________________
	IP ___________________________
	Ethernet ______________________

OTHER

1) (Yes/No) Ethernet retransmits if there is a collision. When I run TCP over Ethernet, does the TCP software still set timers to detect lost packets?

2) Suppose NMU had 6000 laptop computers and only 7 class C addresses.  What protocol could be used to assign IP numbers to laptops?

3) Given the data above, what percent of laptops could be in use at any one time?  SHOW WORK!__________________%

4) Using BGP, how much information does Northern’s routers tell the rest of the internet?
	Just our connections off campus.
	Those, plus the topography of our internal networks.
	That, plus data about every computer attached to our networks.

5) Calculus!! What does this expression evaluate to? ___________

lim x-->2 17

6) Using DHCP, what must a client do if the server refuses to renew a lease and the lease period runs out?

7) My IP number is 192.168.1.103.  What is the host portion of my IP number?  The host portion is the part that says 'which computer within the given network'.

NETWORK ADDRESS TRANSLATION

1) At my house we are using Network Address Translation to access the internet via our cable modem.  When my computer goes to access www.ebay.com, what IP number does it use for the destination on the outgoing packets?
       The cable modem            www.ebay.com            something else        my computer

2) At my house we are using Network Address Translation to access the internet via our cable modem.  When my computer goes to access www.ebay.com, what Ethernet MAC address does it use for the destination on the outgoing packets?
       The cable modem            www.ebay.com               something else       my computer

3) When www.ebay.com replies, what IP address does it use on the reply?
       The cable modem            www.ebay.com               something else       my computer